UNIT -2 :

A digital communication system transfers information from a source to a destination using digital signals. Unlike analog systems, it deals with discrete values (0s and 1s).

1. Block Diagram Components

The system is divided into the Transmitter (Top row), the Channel, and the Receiver (Bottom row).

- Source Digital Information: The input can be Analog (like voice) or Digital (like a keyboard). If analog, it is converted to digital using Pulse Code Modulation (PCM).

- Source Encoder (Compression): Reduces redundancy to minimize the data size.

  - Logic: Assign fewer bits to data that occurs frequently and more bits to data that occurs rarely.

- Channel Encoder (Error Correction): Adds extra "parity" bits to the data. This helps the receiver detect and correct errors caused by noise in the channel.

- Digital Modulation: Converts the binary data into a signal suitable for transmission over an electrical channel (e.g., ASK, FSK, PSK).

- Electrical Channel: The physical medium (wired like Co-axial/Copper or wireless) where noise and interference are introduced.

- Digital Demodulation: The reverse of modulation; it converts the received electrical signal back into binary data.

- Channel Decoder: Uses the extra bits added earlier to find and fix errors.

- Source Decoder: Decompresses the data back into its original "raw" format.

- Destination Digital Information: The final stage where digital data is often converted back to Analog for the user.

2. Analog to Digital Conversion (PCM)

To process an analog signal digitally, it must go through Pulse Code Modulation (PCM), which involves three main steps:

- Sampling: Measuring the analog signal at specific intervals.

- Quantization: Assigning a numerical value to each sample.

- Encoding: Converting those values into binary (0s and 1s).

Nyquist Criteria: To avoid losing information, the sampling frequency ($f_s$) must be at least twice the maximum frequency ($W$) of the message signal:

$$f_s \ge 2W$$

3. Advantages & Disadvantages

| Advantages | Disadvantages |
| --- | --- |
| Noise Immunity: Digital signals are easier to recover from noise than analog signals. | Complexity: The circuits for transmitters and receivers are more complex. |
| Multiplexing: Multiple types of data (voice, video, text) can be combined on one line. | High Bandwidth: Digital signals generally require more bandwidth than analog. |
| Security: Encryption (coding) makes data much harder to intercept. | Synchronization: The transmitter and receiver clocks must be perfectly aligned. |
| Cost: Modern hardware (microprocessors/ICs) is relatively cheap. | Conversion Loss: A/D and D/A conversion can introduce slight inaccuracies. |

4. Real-World Applications

- Telecommunications: Mobile networks (5G/LTE), VoIP (WhatsApp/Skype calls), and Broadband.

- Wireless & Broadcasting: Wi-Fi, Bluetooth, Satellite TV, and Digital Radio.

- Military & Aerospace: Secure communications, Radar systems, and Space exploration.

- Medical: Telemedicine and wireless patient monitoring.

- Banking: Online banking, ATMs, and POS (card) machines.

Summary of Modulation Types

- ASK (Amplitude Shift Keying): Changes amplitude.

- FSK (Frequency Shift Keying): Changes frequency.

- PSK (Phase Shift Keying): Changes phase.

II. Analog to Digital Conversion (PCM)

The conversion process follows three main steps: Sampling $\rightarrow$ Quantization $\rightarrow$ Coding.

1. Sampling (Time Discretization)

Sampling converts a continuous-time signal $m(t)$ into a discrete-time signal $m(nT_s)$.

- Sampling Theorem: To reconstruct a signal, you must sample at the Nyquist Rate: $f_s \ge 2W$ (where $W$ is the bandwidth).

- Nyquist Interval: $T_s = \frac{1}{f_s}$.

- Types of Sampling:

  - Instantaneous: Uses an impulse $\delta(t)$; not practically possible.

  - Natural: Signal is chopped; cannot be easily recognized.

  - Flat-top: Uses a Sample & Hold circuit; the standard practical approach.

- Aliasing: If $f_s < 2W$, the frequency spectra overlap (Aliasing Effect). To prevent this, an Anti-Aliasing Filter (AAF)—essentially an LPF—is used before sampling to limit the signal to $3.5\text{ kHz}$.

2. Quantization (Amplitude Discretization)

Quantization assigns discrete levels to the sampled voltages.

- Step Size ($\Delta$): The difference between two representation levels.

  - For a Uniform Quantizer: $\Delta = \frac{2x_{max}}{L}$, where $L$ is the number of levels.

  - Since $L = 2^n$ ($n$ = number of bits), then $\Delta = \frac{2x_{max}}{2^n}$.

- Quantization Noise ($q_e$): The error $e = m(nT_s) - m_q(nT_s)$.

- Uniform Quantizers:

  - Mid-tread: The origin lies on a "tread" (flat part); the step overlaps at the origin.

  - Mid-riser: The origin lies on a "rise" (vertical part); noise is getting quantized here.

III. Mathematical Derivations (SNR)

1. Signal Power and Noise Power

- Signal Power ($S_P$): Represented as variance $\sigma_x^2$.

  - $\sigma_x^2 = \text{Mean Square} - (\text{Mean})^2$.

  - Assuming zero mean, $\sigma_x^2 = \text{Mean Square}$.

- Quantization Noise Power ($N_P$): Represented as $\sigma_q^2$.

  - $q_e$ is a random variable uniformly distributed between $[-\frac{\Delta}{2}, \frac{\Delta}{2}]$.

  - $\sigma_q^2 = \int_{-\Delta/2}^{\Delta/2} q^2 \frac{1}{\Delta} dq = \frac{1}{\Delta} \left[ \frac{q^3}{3} \right]_{-\Delta/2}^{\Delta/2} = \frac{\Delta^2}{12}$.

2. Signal-to-Noise Ratio (SNR) Derivation

$$(SNR)_o = \frac{\sigma_x^2}{\sigma_q^2} = \frac{\sigma_x^2}{\Delta^2 / 12} = \frac{12 \sigma_x^2}{\Delta^2}$$

Substituting $\Delta = \frac{2x_{max}}{2^n}$:

$$(SNR)_o = \frac{12 \sigma_x^2}{(\frac{2x_{max}}{2^n})^2} = \frac{12 \sigma_x^2}{4x_{max}^2} \cdot 2^{2n} = \frac{3 \sigma_x^2}{x_{max}^2} \cdot 2^{2n}$$

In decibels ($dB$):

$$10 \log_{10}(SNR) = 10 \log_{10} \left( \frac{3 \sigma_x^2}{x_{max}^2} \right) + 20n \log_{10}(2)$$

Based on the notes, for specific signal assumptions, this simplifies to the "6n rule":

$$(SNR)_{dB} = 6n - 7.37$$

Conclusion: Increasing $1$ bit increases SNR by $6\text{ dB}$.

IV. Companding (Non-Uniform Quantization)

Uniform quantization is inefficient for signals with "Weak Passages" (low SNR) and "Loud Passages" (unnecessary accuracy).

- Companding = Compression + Expanding.

- The Process:

  - Compression: Applied at the transmitter using a non-linear (logarithmic) operator. It amplifies weak signals and compresses strong signals.

  - Uniform Quantization: The compressed signal is then sampled and quantized uniformly.

  - Expansion: Applied at the receiver to restore the original signal levels.

- Types of Companding:

  - $\mu$-Law: $\frac{|y|}{x_{max}} = \frac{\ln(1 + \mu |x/x_{max}|)}{\ln(1 + \mu)}$. Standard value $\mu = 255$.

  - A-Law: Used for Europe/International standards.

- Benefit: It creates a Robust Quantizer where the SNR remains constant regardless of signal strength.